TECHNICAL FEATURES CEL-1680 ELECTRIC
Two sections.
With facial opening and plug.
Backrest adjustable to +70° via gas spring.
UPHOLSTERY: wood fiber board covered with 50 mm foam, density 30 kg, upholstered with fire-retardant, antibacterial synthetic leather.
STRUCTURE: epoxy-coated steel tube.
Base with 4 retractable wheels of 75 mm diameter, with locking system and 1 leveling leg. (NOT suitable for transporting people).
Paper roll holder.
APPROXIMATE DIMENSIONS:
Length 1930 mm, width 675 mm.
Height adjustable (470 to 955 mm).
Recommended maximum supported weight: 170 kg.
Weight of the table: 71 kg.
HEIGHT ADJUSTMENT: via electric motor with foot control.
Structure warranty 5 years, motor 2 years.