TECHNICAL FEATURES CEL-1674 AND CHI-1673
3-section examination table.
Backrest adjustable to +75°, leg area to -90° via gas springs.
UPHOLSTERY: wood fiber board covered with 50 mm foam, density 30 kg, upholstered with fire-retardant, antibacterial synthetic leather.
STRUCTURE: epoxy-coated steel tube.
Paper roll holder.
Rotating and height-adjustable arms.
Rotating and height-adjustable leg supports.
Stainless steel drawer for waste.
Base with 4 leveling pads.
APPROXIMATE DIMENSIONS:
Length 1850 mm, width 600 mm, width with leg supports 1025 mm.
CEL-1674 height adjustable (540 to 855 mm) via electric motor with foot control.
CHI-1673 height adjustable (640 to 945 mm) via hydraulic pump.
Recommended maximum supported weight: 170 kg.
Weight of the table: 61 kg.