TECHNICAL FEATURES CFI-1672
Three-section couch.
No face hole.
Backrest adjustable to +30º via ratchet hinge.
UPHOLSTERY: wood fiber board covered with 50 mm foam, 30 kg density, coated with flame-retardant, antibacterial faux leather.
STRUCTURE: steel with epoxy finish.
BASE with stainless steel skirting board AISI 304.
Rotating and height-adjustable leg supports.
Stainless steel waste drawer.
5 metal drawers integrated into the structure with plastic interiors.
Paper roll holder.
APPROXIMATE DIMENSIONS:
Length 1700 mm, width 610 mm.
Fixed height 840 mm.
Maximum recommended weight capacity: 170 kg.
Couch weight: 87 kg.
Structure warranty: 5 years.