TECHNICAL SPECIFICATIONS SIL-1362 1362.A 1362.1 1362.1.A
Three-section chair.
Backrest adjustment from +90 to 0º (lying flat), leg area to -90º, using gas springs with handles on both sides.
Handle on the backrest to push the chair.
Footrest made of MDF with non-slip black mesh, dimensions 320x310 mm.
SIL-1362 padded, chrome-plated, height-adjustable armrests.
SIL-1362.A wooden, chrome-plated, height-adjustable armrests.
UPHOLSTERY: wood fiber board covered with 70 mm foam, 30 kg density + 20 mm viscoelastic foam, coated with fireproof and antibacterial faux leather.
Optional accessory: Height-adjustable IV pole from 750 to 1270 mm.
STRUCTURE: steel tube with epoxy finish.
SIL-1362 and SIL-1362.A with base on 4 plastic wheels of 125 mm. The 2 rear wheels are operated by a centralized brake system activated by double pedals on both sides of the chair. Includes a directional wheel.
SIL-1362.1 and SIL-1362.1.A with base supported on 4 wheels of 100 mm, the 2 rear ones with individual brakes.
APPROXIMATE DIMENSIONS:
Length 867 mm, width 500 mm, width with armrests 718 mm.
Seat height 518 mm.
Chair height 1300 mm.
Recommended maximum supported weight: 150 kg.
Chair weight: 46 kg.
Structure warranty 5 years.