TECHNICAL FEATURES CAM-6099
4 articulated planes divided into 3 parts via motor with handheld control, enabling Fowler, Vascular, Trend, and Anti-Trendelenburg functions.
Beech slatted bed surface. Side rails made of MDF beech. Headboards and footboards made of beech wood and melamine.
STRUCTURE: Internal part of headboards and footboards and bed frame made of epoxy-coated steel tube.
APPROXIMATE DIMENSIONS:
Exterior Length 2065 x 1040 mm. Interior: 1900 x 900 mm.
Backrest section 595 x 790 mm. Fixed section 395 x 790 mm.
Leg section 390 x 790 mm. Foot section 415 x 790 mm.
Backrest adjustable from 0º to 70º, leg section from 0º to 30º.
Trendelenburg 11º, Anti-Trendelenburg 11º.
Height adjustable (370 to 750 mm) via electric motor hidden in headboards and footboards.
Maximum recommended weight: 135 kg.
ELECTRICAL PARAMETERS:
Power supply: 240 V, +/-10 %, 50-60 Hz. Operating voltage: 29 V DC.
Protection against water and dust IPX4. Noise level: < 65 dB.
Electric motor operation: 15 %, max. 2 minutes ON / 12 min OFF.
ACCESSORIES NOT INCLUDED: IV pole, suspension column.
Warranty: structure 5 years, motor 2 years.