TECHNICAL SPECIFICATIONS CAM-6160
4 articulated sections divided into 3 parts, powered by a motor with hand control, enabling Fowler, Vascular, Trendelenburg, and Anti-Trendelenburg functions.
Bed base with beechwood slats.

STRUCTURE:
Steel tubing with epoxy finish.
Base with 4 wheels: 2 with a diameter of 75 mm with brakes on the foot side and 2 unidirectional wheels of 100 mm at the head side.

APPROXIMATE DIMENSIONS:
Length: 1900 mm, Width: 900 mm.
Backrest section: 595 x 790 mm.
Fixed section: 400 x 790 mm.
Leg section: 390 x 790 mm.
Foot section: 415 x 790 mm.
Backrest adjustment: 0º to 70º.
Leg section: 0º to 15º.
Trendelenburg and Anti-Trendelenburg: 15º.
Height adjustable (230 to 730 mm) with straight ascent and no lateral displacement, powered by an electric motor with hand control.
Maximum recommended weight: 135 kg.

ELECTRICAL PARAMETERS:
Power supply: 230 V, +/-10 %, 50-60 Hz.
Operating voltage: 24 V DC.
Water and dust protection: IP66/IP4.
Noise level: < 65 dB.
Motor operating time: 10 %, max. 2 minutes ON / 18 OFF.

ACCESSORIES NOT INCLUDED:
IV stand, suspension column, head and side rails.

WARRANTY:
Structure: 5 years.
Motor: 2 years.