TECHNICAL FEATURES CEL-1675
3-section treatment table.
Adjustable backrest to +75º with gas springs, leg section adjustable manually to -90º.
UPHOLSTERY: wood fiberboard covered with 50 mm foam, density 30 kg, finished with flame-retardant, antibacterial artificial leather.
STRUCTURE: steel tube with epoxy finish.
Base with 4 retractable wheels of 75 mm diameter, with locking system and 1 leveling foot. (NOT suitable for transporting people).
Rotating and height-adjustable leg sections.
Roll holder.
Stainless steel waste drawer.
APPROXIMATE DIMENSIONS:
Length 2000 mm, width 650 mm, width with leg sections 1115 mm.
Height adjustable (600 to 965 mm) via electric motor with foot control.
Recommended maximum supported weight: 170 kg.
Weight of the table: 80 kg.