TECHNICAL FEATURES CAM-6007
4 articulated planes in 3 parts with motor and hand control, allowing functions: Fowler, Vascular, Trend, and Anti-Trendelenburg.
Headboards made of beech melamine.
STRUCTURE: Steel tube, epoxy finish.
Base with 4 double swivel wheels, 75 mm in diameter, with brake.
APPROXIMATE DIMENSIONS: Outer length 2250, outer width 920 mm.
Inner length 2000, inner width 900 mm.
Backrest section 700x790 mm. Fixed section 250x790 mm.
Leg section 350x790 mm. Foot section 590x790 mm.
Backrest adjustable from 0º to 70º, leg section from 0º to 38º.
Trend adjustment 15º, Anti-Trendelenburg 15º.
Height adjustable (70 to 650 mm.) by electric motor with hand control.
Recommended maximum weight: 135 kg.
ELECTRICAL PARAMETERS:
Power supply: 230 V, +/-10%, 50-60 Hz. Operating voltage: 24 V DC.
Water and dust protection IP66/IP4. Noise level: < 65 Db.
Electric motor operation: 10%, max. 2 minutes/18 minutes.
ACCESSORIES NOT INCLUDED: IV stand, suspension column.
Warranty structure 5 years, motor 2 years.