TECHNICAL FEATURES CAM-6068.DOMUS
4 articulated planes in 3 parts with motor and manual control.
Bed with beech slats.
Set of headboards and side rails made of MDF with beech finish.
STRUCTURE: Steel tube with epoxy finish.
Base with 4 wheels of 100 mm diameter, two with brake.

APPROXIMATE MEASUREMENTS: Length 1900 mm, width 900 mm.
Back section 595 x 790 mm. Fixed section 400 x 790 mm.
Leg section 390 x 790 mm. Foot section 415 x 790 mm.
Backrest adjustable from 0° to 70°, leg section from 0° to 15°.
Height adjustable (410 to 800 mm) straight lift without lateral displacement, with electric motor and manual control.

Recommended maximum weight: 135 kg.

ELECTRICAL PARAMETERS:
Power supply: 230 V, +/-10 %, 50-60 Hz. Operating voltage: 24 V DC.
Noise level: <65 dB. Protection against water and dust IP66/IP4.
Electric motor operation: 10 %, max. 2 minutes/18 minutes.

ACCESSORIES NOT INCLUDED: Serum holder and suspension column
Warranty: structure 5 years, motor 2 years.