Note: The image shown is a demonstration and does not fully correspond to the basic model, which does not include some of the options shown in the image.

TECHNICAL CHARACTERISTICS CAM-6500

4 articulated planes in 3 sections. Electric lifting of the backrest, legs, bed, Trend, and anti-Trendelenburg, autocontour 30°/45°, and cardiac chair. Mechanical lifting of the foot section.
Removable Plus model headboard, easy to disassemble (no tools required).
ABS side rails. 4 bumpers with a diameter of 100 mm.
Double self-regression to reduce abdominal pressure in the backrest and leg sections. Anti-crushing system in the backrest and leg sections.
Removable bed in double plastic panels (polypropylene), for easy cleaning and disinfection.
Emergency release in the backrest section.
Urine bag holder made of stainless steel on both sides of the bed.
Possibility to attach the IV pole and trapeze to the 4 corners of the bed.

STRUCTURE: Steel tube with epoxy finish.
Base with 4 swivel wheels of 150 mm in diameter with directional centralized brake operated by 2 double pedals accessible from both sides of the bed. 1 of the wheels is electrically conductive.
Individual function lock on the remote control (key).

APPROXIMATE DIMENSIONS: Inner length 2180 mm, inner width upper rail 980 mm, lower rail 960 mm.
Backrest section 820 mm. Fixed section 340 mm.
Leg section 340 mm. Foot section 500 mm.
Backrest adjustable from 0° to 70°, leg section from 0° to 30°, foot section from 0° to 25°.
Trend and anti-Trendelenburg adjustment +17° / -17°.
Height adjustable (385 to 845 mm) by electric motor with manual control.
Clearance under the base structure 170 mm.

Recommended maximum load: 220 kg.

ELECTRICAL PARAMETERS:
Power supply: 100/230 V AC - 50-60 Hz. Operating voltage: 24 V DC.
Voltage control system: 5 V DC.
Protection against solid and liquid particles: IP66.
Electric battery for autonomous movements (100 movements).

OPTIONAL ACCESSORIES NOT INCLUDED:
ACC-1163: Traction cable support set.
ACC-1164: Trapeze support in stainless steel.
ACC-1166: IV pole in stainless steel.
ACC-1167: Support for monitor/desk - Headboard mod. I and II.
ACC-1168: Support for oxygen bottle.
ACC-1169: A-4 desk/acrylic board.
ACC-1170: Patient identification plate - plastic headboard Model I/Plus.
ACC-1173: Anti-decubitus hospital mattress, 1950 x 840 x 150 mm.
ACC-1174: Anti-decubitus hospital mattress, 1980 x 880 x 150 mm.
ACC-6500.A: Side rail with integrated control - inside for patient, outside for professional.
ACC-6500.B: Battery.
ACC-6500.D: Double wheel 150 mm.
ACC-6500.E: Night light.
ACC-6500.F: Extendable clothing rack.
ACC-6500.G: Extension for mattress support in the foot section.
ACC-6500.H: In-situ function expansion: Exit, vascular, shock.
ACC-6500.I: Hand remote HB85.
ACC-6500.J: ACO foot control - basic (includes clothing rack).
ACC-6500.K: 5th directional wheel.
ACC-6500.L: 5th single wheel.
ACC-6500.M: Double pedal - 4 functions (up, down, cardiac chair, exam position).
ACC-6500.N: Single pedal - 2 functions (up, down).
ACC-6500.O: Brake alarm.
ACC-6500.P: Emergency release - foot section.
ACC-6500.Q: Metallic side rails.
ACC-6500.R: Fixed headboard at the base.
ACC-6500.S: Fixed HPL bed.
ACC-6500.U: RX HPL support - back section.
ACC-6500.V: Urimeter rail in stainless steel.
ACC-6500.X: Plastic cover for the base structure.
ACC-6500.Z: Patient detection sensor.

Warranty: structure 5 years, motor 2 years