TECHNICAL FEATURES CAM-HC02 / CAM-HC02.3 / CAM-HC04
4 articulated planes in 3 parts powered by a motor with manual control, allowing Fowler, vascular, semi-lying, and leg flexion positions.
Bed with beech slats.
Headboards with wood molding and melamine interior. Rails made of laminated MDF.
STRUCTURE: Steel tube with epoxy finish.
CAM-HC02: Fixed height with 4 stainless steel legs.
CAM-HC02.3: Fixed height with 4 swivel double wheels of 75 mm diameter, 2 of them with brakes.
CAM-HC04: Height adjustable (410 to 800 mm) via electric motor with manual control. Base with 4 wheels of 100 mm diameter, 2 with brakes.
APPROXIMATE MEASUREMENTS: Length 1900 mm, width 900 mm.
Back section 595 x 790 mm. Fixed section 400 x 790 mm.
Leg section 390 x 790 mm. Foot section 415 x 790 mm.
Backrest adjustable from 0° to 70°, leg section from 0° to 15°.
Recommended maximum weight capacity: 135 kg.
ELECTRICAL PARAMETERS:
Power supply: 220 V, +/-10 %, 50-60 Hz. Operating voltage: 24 V DC.
Noise level: <65 dB. Protection against water and dust IP44.
Electric motor operation: 10 %, max. 2 minutes/18 minutes.
Warranty: structure 5 years, motor 2 years