TECHNICAL FEATURES SIL-1380

Chair designed for comfortably positioning patients for blood sampling, pain management, dialysis, oncology therapies, and other similar treatments.

Chair with 3 sections adjustable via 3 motors operated by a hand controller.
Headrest cushion.
UPHOLSTERY: 16 mm MDF board, backrest and seat covered with 50 mm foam, density 30 kg, plus 200 mm viscoelastic foam, armrests and leg rests 23 kg, covered with fire-retardant, antibacterial faux leather.
Adjustable side arms allowing 90° lateral rotation and 160° vertical rotation via gas springs.
Shock or Trendelenburg position can be reached automatically, as well as bed position using a supervisory control (optional, not included).
STRUCTURE: Steel tube base with epoxy finish, PS polystyrene cover.
Base with 4 wheels Ø 125 mm, with centralized and directional brake system operated by 2 double pedals, accessible from both sides of the chair.
APPROXIMATE DIMENSIONS:
Exterior: reclined length 2055 mm. Width 970 mm. Interior: width 600 mm.
Backrest section 890x600 mm. Seat section 560x600 mm.
Leg section 605x600 mm.
Armrests 580x145 mm.
Backrest adjustable from 0° to 90°, seat from 0° to 25°, leg section from 0° to 45°.
Trendelenburg adjustment 15°.
Height adjustable (600 to 850 mm), straight rise without lateral shift, via electric motor with hand control.
Recommended maximum load: 200 kg.
Chair weight: 110 kg.
ELECTRICAL PARAMETERS:
Power supply: 230 V, +/-10%, 50-60 Hz. Operating voltage: 24 V DC.
Protection against water and dust IP54. Noise level: <65 dB.
Electric motor operation: 10%, max. 2 minutes/18 min.
OPTIONAL ACCESSORIES NOT INCLUDED:
ACC-1380.A (Roll holder and handle) / ACC-1380.B (IV pole and holder).
ACC-1380.C (Document holder) / ACC-1380.D (Battery).
ACC-1380.E (Night light) / ACC-1380.F (USB connection).
ACC-1380.G (Supervisory control) / ACC-1380.H (Emergency button).
ACC-1380.I (Footrest system).

Warranty structure 5 years, motor 2 years.